# Column RFT Detailing Spec (v1)

# Column RFT Detailing Spec (v1 — Single Floor-to-Floor Segment)

Status: LOCKED — scope, detailing rules, and citations are final. This document authorizes exactly one next step: the API tracer bullet in §11. No core module code or UI work is started by this document.

Companion to beam-rft-detailing.md. Written the same way: every rule cites its source, every open question is answered in writing rather than left to be guessed at build time, and every deliberate deviation from code is named as an exception rather than left silent.

## 0. Scope

In scope: detailing the longitudinal bars and transverse reinforcement (ties/stirrups + cross-ties) of ONE column segment, floor-to-floor, for a single rectangular column of user-input dimensions a x b.

Out of scope, explicitly (not “later,” named as cut now):

- C1 — Foundation starter bars / dowels. Bars protruding from a footing into the base of a ground-floor column are out of scope. A separate footing tool will own that connection. This tool’s bottom boundary is “continues the bar from the segment below” (see §6), never “starts from a footing.”

- C2 — Multi-story continuity beyond one splice. This tool details exactly one floor-to-floor segment and its top-of-support splice protrusion into the segment above (§6). It does not model a full-height column stack across multiple stories in one run.

- C3 — Non-rectangular columns. Circular, L-shaped, or other cross-sections are out of scope for v1.

## 1. Inputs (user-provided, never invented)

| Input | Notes |
| --- | --- |
| a_mm, b_mm | Column cross-section dimensions. |
| Hc_mm | Clear height of the column segment (support face to support face). |
| Longitudinal bar type, count per face, corner bars | Ø and fy read from the selected RebarBarType, same convention as beam’s role_picker_label. |
| Tie/stirrup bar type | Ø and fy, user-selected. |
| Cover | Cover to ties/stirrups. Assumed 25 mm as a starting default per the |

original sketch, but must remain an editable input, not a hardcoded constant — same discipline as the beam tool’s per-face cover reads. | | Main outer tie hook type | User-selected dropdown, populated from available RebarHookTypes in the project, default 135° (§7). | | Inner tie hook type | A SEPARATE, independently user-selected dropdown from the main outer tie hook, default 135° (§7). | | Spacing mode | Toggle: Mode A (Auto-Calculate) or Mode B (Manual Override) — see §8. | | L_s (splice length) | Raw user input (value or diameter multiplier). NEVER auto-calculated by the tool (§6). |

## 2. Data model: Perimeter Layout (C9)

Decision: the column’s longitudinal bar layout is modeled as ONE continuous perimeter arrangement (corner bars + intermediate bars per side), not as independent faces.

This is a deliberate departure from the beam tool’s FacePlan/LayerPlan model. A beam’s two faces (top/bottom) are genuinely independent — different bar counts, different layer counts, different anchorage per face. A column’s longitudinal bars, ties, confinement zone, and splice rule all act on ONE system around the full perimeter at once. Force-fitting the beam’s two-face model onto four column faces would misrepresent the geometry and was rejected on that basis.

Consequence: this warrants a new core module (working name rft.core.column_layout, final name TBD at implementation), not a re-skin of rft.core.layout. Do not assume beam’s per-face functions apply without re-deriving them against the perimeter model.

Governing dimension rule (C2, closes the original a/b question):

Evaluate both cross-section dimensions a and b. The SMALLER dimension governs S₀. The LARGER dimension governs L₀. Both governing values are then applied uniformly to all four faces of the column — independent per-axis values are explicitly rejected, by decision, in favor of one governing value applied uniformly (standard construction practice, avoids site errors).

## 3. Confinement zone length — L₀

L₀ = max(clear height / 6, larger cross-sectional dimension of the column, 500 mm), measured from the face of the support.

Source: ECP 203, Chapter 8 — Seismic Detailing Provisions. See §12.

## 4. Confinement tie spacing — S₀

S₀ equals the smallest of: 1. 8 × diameter of smallest longitudinal bar 2. 24 × diameter of the tie 3. Half the SMALLER column cross-sectional dimension 4. 150 mm

Source: ECP 203, Chapter 8 — Seismic Detailing Provisions (originally sketched from the manual notes as “13-4-أ”, items 1-4). See §12.

First tie placement (closes the inequality ambiguity):

The first tie is placed at an EXACT offset of 50 mm from the face of the support. This is a definitive placement instruction, replacing the code’s “not exceeding S₀” language, which is a maximum and not by itself a placeable position.

## 5. Middle-zone spacing

Outside L₀, transverse reinforcement spacing shall not exceed 2 × S₀.

This closes what was originally sketched as a bare “150mm” figure — that number belonged to a DIFFERENT rule (§6) and is not an independent cap on middle-zone spacing.

## 6. Horizontal bar restraint & inner tie geometry (C7 — IN SCOPE, AMENDED)

AMENDMENT to the original version of this section: the original simple “1-leg cross-tie” assumption is DELETED. ECP standard practice predominantly restrains interior bars with OVERLAPPING CLOSED TIES (boxes within boxes), not single-leg cross-ties. Everything below replaces the earlier cross-tie language; rft.core.stirrups’ single-closed-rectangle model is even further from what this section now requires than originally scoped.

### 6.1 Which bars need restraint (tiered spacing rule — UNCHANGED)

This folds and REPLACES the earlier loose “150mm horizontal spacing” note. There is no separate hard 150mm cap sitting underneath this rule — this tiered rule is the only horizontal-spacing provision in scope, and it now governs which bars a tie’s SUBSET must enclose, not merely whether a single-leg cross-tie exists.

Evaluate the horizontal clear distance x between consecutive longitudinal bars around the perimeter: - Maximum distance between any two tie branches must not exceed 300 mm. - If x ≤ 150 mm: alternate — tie one bar, leave one bar untied. - If 150 mm < x ≤ 250 mm: every single bar must be tied (restrained by a tie corner or an inner tie leg).

Source: Egyptian Detailing Guide (2001), Figure 13-3 (see §12).

### 6.2 Data model — subset-based closed loops (CADS Rebar logic)

Internal ties are NOT modeled as single-leg cross-ties. The perimeter layout module (rft.core.column_layout, §2) must define each tie — outer and inner alike — using SUBSET logic: an initial bar index and the number of tied bars it encloses, in a given direction (matching the convention used by CADS Rebar Extensions). The module generates CLOSED RECTANGULAR LOOPS around the specified bar range, not open lines.

Example: Tie 1 wraps the full outer perimeter (all bars). Tie 2 wraps only the intermediate bars on the North and South faces. A column’s full tie set is the union of one or more such closed, possibly overlapping, loops — never a mix of “the main tie” plus separate free-floating cross-tie legs.

Open item, not yet decided — flagged rather than assumed: is the subset assignment (which ties enclose which bar ranges, i.e. the column’s tie TOPOLOGY) derived automatically from §6.1’s tiered rule for a given bar layout, or does the tool ask the user to pick/confirm a topology (e.g. select one of Figure 13-3’s typical sections)? Auto-deriving a closed-loop topology from a spacing rule alone is a non-trivial geometry problem; deferring this to the tracer bullet / early implementation spike (§11) rather than guessing now.

### 6.3 Hook corner alternation (NEW rule — do not confuse with §6.1’s

static bar-membership rule)

To avoid a vertical plane of weakness under compressive load, the HOOK CORNER of any closed tie (outer or inner) must alternate at each spacing level, vertically. If a rectangular tie’s hook sits at the North-East corner at level n, it must rotate to a DIFFERENT corner (e.g. North-West) at level n+1. This is built into the vertical array loop, not a one-time placement decision.

This is explicitly a DIFFERENT property from §6.1’s “static pattern” rule, and the two must not be conflated:

|  | What alternates | Vertically? |
| --- | --- | --- |
| §6.1 (bar membership) | WHICH bars are enclosed by a tie | NO — fixed once, applied at every level |
| §6.3 (hook rotation) | WHICH CORNER the tie’s hook sits at | YES — rotates at every level, by design |

Both rules govern the same tie objects; they are not in tension with each other, but a future reader skimming for “does this alternate” must not merge them into one answer.

Consequence: this is new geometry the beam tool never needed at all — subset-defined closed loops, per-level hook-corner rotation, and overlapping tie shapes are a genuinely new module, not a re-skin of rft.core.stirrups.

## 7. Hook types — two independent dropdowns (C8, CONFIRMED)

The main outer tie hook type and the inner tie hook type are TWO INDEPENDENT, user-selectable dropdowns, each populated from the RebarHookTypes available in the project — the same hook_type_options-style approach as the beam tool, but NOT collapsed into one control.

Default value: 135° for both dropdowns. The default is not a hard requirement — the user may change either dropdown to any available RebarHookType — but 135° is what each control opens with, reflecting standard seismic practice, per §6.3’s hook-alternation rule applying to whichever hook type is ultimately selected.

Rationale (stated, not left implicit): inner ties in seismic detailing commonly require a strict 135° hook on at least one end, specifically because an inner tie’s function is restraining an intermediate bar against buckling; the main outer tie may legitimately be detailed differently per project. The tool does not hardcode either angle — it defaults both to 135° and lets the user override, but keeps the two choices structurally separate so one dropdown’s selection can never silently apply to the other role.

## 8. Spacing input: Mode A / Mode B toggle

The UI provides a toggle for vertical tie spacing (S₀ and middle-zone spacing): - Mode A (Auto-Calculate): the tool computes the strict code maximums per §3-§5. - Mode B (Manual Override): the user enters exact spacing values for the confinement zone and the middle zone directly.

Manual-override behavior — “warn but place,” stated explicitly so it is never confused with either silent refusal or silent compliance:

If Mode B’s user-entered spacing violates the code-calculated limit, the tool does NOT refuse, and does NOT silently accept it either. The Review report MUST display the code-calculated limit ALONGSIDE the user’s manual value as a visible flag/warning. The placer logic ultimately respects and builds the user’s manual value regardless of the warning.

This mirrors the beam tool’s single-source-of-truth principle: the Review report and the placer must show/use the same number, so a violation is visible on the page the same instant it becomes true in the model — never discoverable only after the fact.

## 9. Splice location — constructability override (C4, named exception)

Constructability Override (Exception to Seismic Clause): Despite the L₀ confinement zone being a critical seismic region, lap splices shall deliberately start at the top face of the support (falling entirely within the lower L₀ zone of the upper segment). This is a conscious deviation from the mid-height splice rule to accommodate standard real-world floor-by-floor concrete pouring joints. The tool knowingly places lap splices inside L₀ at the base of every story.

Bar geometry: the longitudinal bar is modeled starting from the current floor level, extending upward through the column’s clear height and through the top support, protruding by a splice length L_s above the top support’s face to connect with the next story.

L_s as user input, never auto-calculated:

Because L_s varies based on tension/compression zones, L_s is a direct user input (raw value or diameter multiplier). The tool does not auto-calculate L_s; it blindly applies the user’s input.

## 10. Explicit non-reuse

This tool does NOT call or reuse rft.core.anchorage. Column bar ends are governed entirely by the floor-to-floor splice/protrusion logic in §9, not by beam-style supported/unsupported end anchorage.

Stated plainly, the same way any other non-reuse should be, so a future contributor does not assume anchorage.py should be wired in by default.

## 11. Prerequisite before any core logic or UI is built

A throwaway tracer-bullet script MUST be written and verified against a real Revit host, on a real test column, for: 1. Section orientation — reading which direction is a vs b in the model. A column is a point-based FamilyInstance with a vertical Location.Point; its cross-section rotation does not come from Location.Curve-based axis reads the way beam geometry does, and this exact class of “wrong reference” assumption has cost this project multiple release candidates before. 2. Finding vertical framing neighbours — locating the supporting/ framing element at BOTH the base and the top of the column segment. Beam’s find_supporting_element searches along one line at one end; a column needs a genuinely new vertical search, not a reuse.

Both must be confirmed on a live host BEFORE any core.column_layout module or WPF window is designed around an assumption about either.

## 12. Citations (CLOSED)

| Rule | Source |
| --- | --- |
| §3 — L₀ confinement length | ECP 203, Chapter 8 (Seismic Detailing Provisions). |
| §4 — S₀ confinement spacing, first-tie offset | ECP 203, Chapter 8 (Seismic Detailing Provisions). |
| §6 — horizontal restraint tiers, inner tie subset/closed-loop geometry | Egyptian Detailing Guide (2001), Figure 13-3 (Typical Column Reinforcement Sections). |

This closes the citation gap flagged in the prior draft. All three sources are chapter/figure-level references rather than a numbered sub-clause — if a more granular clause number exists within Chapter 8 or Figure 13-3 for any individual rule (e.g. the exact sub-item for the 8×/24×/half-b/150mm list in §4), it should still be added when convenient, the same standard the beam spec holds itself to (rev 2 section 2.4, not just “rev 2”). This is a refinement to make later, not a blocker to starting the tracer bullet.